# PN532 Embedded Team Guide

NFC Door Access — iPhone Integration

Updated: March 2026 — v3 (Dual-Mode Loop)

# 1. What the Reader Must Do

The PN532 reader has TWO jobs:
• Read physical NFC hotel cards (Reader Mode)
• Receive data from iPhone app (Target Mode)

It CANNOT do both at the same time. It must rapidly switch between the two modes using a Dual-Mode Loop.

# 2. Why Dual-Mode Is Required

Apple iPhone has a hardware-level rule that CANNOT be changed by any app:

• When iPhone detects an external RF field → iOS opens Apple Wallet
• When iPhone generates RF and finds a tag → iOS routes to the app

If the PN532 stays in Reader Mode (generating RF), the iPhone will ALWAYS open Wallet instead of the door access app. No iOS code can prevent this.

The PN532 must be in Target Mode (silent, no RF) when the iPhone taps. The iPhone generates the RF field and the PN532 responds as a tag.

# 3. The Dual-Mode Loop

The PN532 runs a continuous loop that alternates between two modes:

|  | Target Mode (for iPhone) | Reader Mode (for Cards) | Notes |
| --- | --- | --- | --- |
| Duration | 500ms | 20ms | Target mode is 96% of the time |
| RF Field | OFF — PN532 is silent | ON — PN532 generates RF | iPhone only safe when RF is OFF |
| PN532 Command | TgInitAsTarget (0x8C) | InListPassiveTarget (0x4A) |  |

Loop order — Target mode runs FIRST:

  1. Enter Target Mode (500ms timeout)
     → If iPhone detected → stay in Target Mode → receive full NDEF data → loop back
     → If timeout (no iPhone) → go to step 2

  2. Enter Reader Mode (20ms timeout)
     → If NFC card detected → stay in Reader Mode → read card → loop back
     → If timeout (no card) → go to step 1

Target mode is checked FIRST because it is safe (no RF). Reader mode is checked SECOND and kept very short (20ms). A physical NFC card responds in under 5ms, so 20ms is more than enough.

# 4. Why Target Mode Must Be First and Longer

| Scenario | What Happens | Result |
| --- | --- | --- |
| iPhone taps during Target Mode (96% of time) | PN532 is silent (no RF). iPhone generates RF. PN532 responds as tag. | ✅ App receives data |
| iPhone taps during Reader Mode (4% of time) | PN532 is generating RF. iPhone detects external RF field. | ❌ Wallet opens (user retries) |

With 96/4 split, the chance of Wallet opening is very small. If it does happen, the user simply closes Wallet and taps again — next attempt will almost certainly hit the Target Mode window.

# 5. PN532 Target Mode Configuration

When entering Target Mode, the PN532 must present itself as an ISO 14443-4 tag that the iPhone recognizes as an NDEF Type 4 Tag.

## Required Settings:

| Setting | Value | Why |
| --- | --- | --- |
| SAK Byte | 0x20 | Tells iPhone this is ISO 14443-4 compliant |
| ATQA | 0x0004 | Standard Type 4 Tag response |
| NDEF AID | D2 76 00 00 85 01 01 | iPhone sends SELECT for this AID first |
| Historical Bytes | Optional | Can be empty or contain version info |

# 6. What iPhone Sends (When in Target Mode)

When the iPhone detects the PN532 as a tag, it sends these commands in order:

| Step | iPhone Sends | PN532 Must Respond |
| --- | --- | --- |
| 1 | SELECT NDEF AID 00 A4 04 00 07 D2760000850101 00 | Success: 90 00 |
| 2 | SELECT CC File 00 A4 00 0C 02 E103 | Success: 90 00 |
| 3 | READ CC File 00 B0 00 00 0F | Return 15 bytes: 00 0F 20 00 FF 00 FF 04 06 E1 04 04 00 00 00 |
| 4 | SELECT NDEF File 00 A4 00 0C 02 E104 | Success: 90 00 |
| 5 | UPDATE BINARY (write NDEF data) 00 D6 00 00 LEN [data] | Success: 90 00 Store the NDEF data! |

# 7. Data the iPhone Writes

The iPhone writes an NDEF Text Record containing JSON:

{
    "masterKey": "ABCD1234",
    "roomNumber": "405",
    "checkIn": "2026-03-01",
    "checkOut": "2026-03-05"
  }

This is wrapped in NDEF format:
  [NDEF Length (2 bytes)] [NDEF Header] [Status Byte + "en" + JSON text]

The PN532 should:
  1. Extract the JSON text from the NDEF record
  2. Parse the JSON fields
  3. Use masterKey + roomNumber to verify access
  4. Check current date is between checkIn and checkOut

# 8. How iPhone Writes (3-Step Process)

The iPhone uses the NFC Forum Type 4 Tag write procedure:

  Step 1 — Invalidate: Write length = 0000 at offset 0
           (tells reader: data is being written, not valid yet)

  Step 2 — Write Data: Write actual NDEF bytes at offset 2+
           (the guest access JSON data)

  Step 3 — Finalize: Write real length at offset 0
           (tells reader: data is now complete and valid)

The PN532 should only process the data after Step 3 (when the length at offset 0 changes from 0000 to the real value).

# 9. Quick Reference Card

| Item | Value |
| --- | --- |
| PN532 Mode for iPhone | TgInitAsTarget (0x8C) — Target Mode |
| PN532 Mode for Cards | InListPassiveTarget (0x4A) — Reader Mode |
| Loop Timing | Target 500ms → Reader 20ms → repeat |
| SAK Byte | 0x20 |
| NDEF AID | D2 76 00 00 85 01 01 |
| CC File (15 bytes) | 00 0F 20 00 FF 00 FF 04 06 E1 04 04 00 00 00 |
| Data Ready Signal | NDEF length at offset 0 changes from 0000 to real value |

# 10. Troubleshooting

| Problem | Cause | Fix |
| --- | --- | --- |
| Apple Wallet opens instead of app | PN532 is in Reader Mode when iPhone taps (generating RF) | PN532 must be in Target Mode. Use dual-mode loop with Target first. |
| iPhone shows "No tag found" | SAK byte is not 0x20 | Set SAK = 0x20 in TgInitAsTarget parameters |
| iPhone connects but write fails | CC file missing or wrong bytes | Return exact 15 bytes: 00 0F 20 00 FF 00 FF 04 06 E1 04 04 00 00 00 |
| Data received but JSON parse fails | Not stripping NDEF header/status byte | Skip NDEF header + status byte + "en" prefix to get JSON |